# Week 6: Actuators and motion control

Actuators are the parts of a robot that move it. Most small robots use electric motors, because they are cheap, quiet and easy to control from a microcontroller.

A DC motor spins continuously when a voltage is applied. Its speed is set with pulse width modulation (PWM): the motor is switched on and off very fast, and the share of time it is on decides the average speed.

A servo motor turns to a set angle and holds it there. Inside it has a small DC motor, a gearbox and a potentiometer that tells the controller where the shaft is.

A stepper motor moves in fixed steps, often 1.8 degrees each. Because every step is the same size, the controller can work out the position by counting steps, without a sensor.

An H-bridge lets a microcontroller drive a DC motor in both directions. It is a circuit of four switches that can reverse the voltage across the motor.

Encoders measure how far a wheel has turned. Counting encoder ticks gives odometry, an estimate of how far the robot has travelled, which slowly drifts because wheels slip.

A PID controller keeps a value close to a target. The proportional part reacts to the current error, the integral part removes steady error over time, and the derivative part damps overshoot.

Gear ratios trade speed for torque. A 10:1 gearbox makes the output shaft turn ten times slower but with about ten times the torque.